Obtaining and Activating Product License (Security) Keys for Pagent and Callgen Images:

1.	What are Pagent and Callgen?

Pagent is a set of data networking test tools, based on the Cisco IOS (Internetwork Operating System) and developed within Cisco. The test tools are included in special IOS Pagent images.

Callgen is a set of Voice-Over-IP networking test tools, based on the Cisco IOS and developed within Cisco. The test tools are included in special IOS Callgen images.

The primary function of the Pagent and Callgen toolsets is to provide cost effective test tools to the Cisco testing community. The secondary function is to provide testing capabilities that are not available anywhere else and can be used in customer environments. One essential function is to generate valid simulated traffic necessary for testing networks under stress. The tools are not able to test the physical network layer and some parts of the data-link layer; that is, they can not affect frame checksums, preambles, inter frame gap times, inject hardware failures, or transmit invalid packet sizes.

The Pagent programs are best used for testing layer 3 protocols and above; that is, emulating routing protocols, multicast, TCP sessions and HTTP sessions.

Some newer certification labs may contain one or more Cisco router platforms dedicated to the Pagent and/or Callgen tool sets (see lab diagram in Lab Guide or Course Administration Guide).

2.	Why are License Keys needed for Pagent and Callgen?
Pagent and Callgen images have a security scheme to prevent illegal distribution outside Cisco. When a router is loaded with a Pagent image for the first time, it presents a Machine ID that must be used to generate a license key by a Cisco TIS lab administrator.

3.	How are the Pagent and Callgen license keys activated and for how long are they valid?

When installing a lab for the first time, or anytime there is a need to replace a Pagent or Callgen router platform, you or your lab administrator will have to request a new license key for each router platform that is replaced or newly installed. Once the license key is entered in the router, it is saved in the IOS configuration so it is not required on subsequent downloads.

During first time a secure image is loaded, the machine ID will be displayed on the console screen when a user requests entering the enable mode. Get the machine ID number and e-mail it to the TIS lab administrators:

tis_support@cisco.com

You will need to supply in your email request:

- router platform

- Machine ID listed on the console after the OS Image is loaded

- CLP Name

- the purpose of using Pagent and/or Callgen

TIS lab admins will verify your CLP affiliation and e-mail the keys which you can then load and activate in the proper platform. Please note that the key will be saved in the configuration stored in NVRAM after successful validation.

Once the keys are validated and activated, you do not have to repeat the process unless you need to replace a Pagent or Callgen router.